Circular Motion

- Terms and Symbols:

- Radius(r) = Distance from the center of circular motion.

- Change in Time (Δt)  = Change in time.

- Arc length (s)  = Distance measured along the circumference of a circle.

- Angle (θ) = Angle of rotation measured in Radians (rad)

- Radian(rad) = An angle whose arc length (s) is equal to its radius (r). It is approximately 57.3˚

θrad = s

r

- 1 complete circle has 2π radians.

- Circumference (c) = Distance around the outside track of a circle.

c=2πr

- Angular Speed (ώavg)  and Angular Acceleration (αavg)

- Must use radians (rad)

θrad =  π       x  θdeg

180˚

- Angular Displacement (Δθ) -Describes how much an object has

rotated.

Δθ = θf – θi

Δθ = Δs

r

Units: Radians(rad)

- Positive for counterclockwise rotation

- Negative for clockwise rotation

- Angular Speed (ώavg) - Describes rate of rotation: (Greek letter Omega)

ώavg = Δ θ    =    θf – θi

Δ t           tf - ti

Units: rad/s

- sometimes measured in rpm   1rev=2πrad

- Angular Acceleration(αavg) -occurs when angular speed(ώavg)

changes.

αavg = ώf – ώi  = Δώ

tf – ti        Δt

Units: rad/s2

- Relationship between angular and linear variables

| Variable | Linear | Angular | Relationship |
| --- | --- | --- | --- |
| Displacement | x (m) | θ (rad) | s=θr |
| Velocity | v (m/s) | ώ (rad/s) | vt=ώr |
| Acceleration | a (m/s2) | α (rad/s2) | at=αr |

- Rotational and Linear Kinematics Equations

| Rotational motion | Linear motion |
| --- | --- |
| ώf = ώi + αt | vf = vi + at |
| Δθ = ώit + 1/2αt2 | Δx = vit + 1/2 at2 |
| ώf2 = ώi2 + 2αΔθ | vf2 = vi2 + 2aΔx |

- Linear Speed and Linear Acceleration

- Tangential Speed (Vt) – The instantaneous linear speed of an

object directed along the tangent to the

object’s circular path.

Vt = rώ

Units: m/s

- ώ must be in rad not rpm

- Tangential acceleration (at) –The instantaneous linear

acceleration of an object directed

along the tangent to the object’s

circular path.

at = rα

Units: m/s2

- Centripetal Acceleration (ac) – Acceleration directed toward the

center of a circular path.

ac = vt2

r

ac = rώ2

Units: m/s2

- Tangential and centripetal accelerations are perpendicular to each other. The total acceleration is given by the Pythagorean Theorem.

atotal2 = at2 + ac2

The force that maintains circular motion is Fc and counteracts the inertia of the object in circular motion wanting it to follow a straight line. The force is directed toward the venter of rotation.

Fc = mvt2

r

Fc = mac

Fc = mrώ2

Units: Newtons